IN THE UNITED STATES PATENT AND TRADEMARK OFFICE

Provisional Utility Patent Application

TITLE:	DART DELIVERY SYSTEMS AND METHODS FOR OIL AND GAS WELL COMPLETIONS

INVENTORS:	Samuel Caspary, Houston, Texas, Reid Caspary, Houston, Texas and Raymond McNeil, Houston, Texas

ASSIGNEE:		Sharewell HDD LLC, Houston, Texas

BACKGROUND OF THE INVENTIONS

Field of Use:

[0001] The present invention relates to oil and gas well completion technologies, and more particularly to delivery systems, including ground-based and aerial delivery systems for deploying downhole components such as darts, balls, or other tools into a wellhead using robotic and/or remote controlled systems, suspension cable conveyance and launch systems and/or unmanned aerial vehicles (UAVs). In preferred embodiments, these systems facilitate safe and efficient deployment of downhole components during hydraulic fracturing, replacing traditional plug-and-perf operations where humans are operating in the “red zone” of a wellhead, and where such tools are essential for actuating downhole mechanisms.

Description of Related Art

[0002] In the oil and gas industry, hydraulic fracturing (commonly known as fracking) involves injecting high-pressure fluids mixed with proppants and chemicals into subterranean formations to create fractures in the rock, thereby enhancing the release and flow of hydrocarbons such as oil or natural gas. Traditional completion operations utilize the “plug-and-perf” completion method which entails lowering a wireline down the completion casing and setting off perforating guns with high explosives to create holes in the casing to access the formation.  A  key and more recent method in multi-stage fracking, requires the sequential delivery of darts or balls into a wellbore to actuate downhole perforating sleeves, such as frac sleeves. Frac sleeves are downhole tools integrated into the casing string that isolate sections of the wellbore; They are traditionally activated by wireline and/or coiled tubing operations, which are costly and time consuming.  More recent innovations to activate frac sleeves include dropping a “smart” dart or ball that seats in the desired frac sleeve, generating pressure to shift it open for fluid injection and fracking into the formation.  Compared to traditional perforating guns—which require wireline or coiled tubing runs to create perforations in the casing—frac sleeves offer significant advantages, including the ability to perform multi-stage fracturing in a single trip downhole, reducing operational time, costs, and risks associated with repeated tool deployments. For purposes of this description, these actuating devices are referred to as darts, though the systems discussed may also apply to balls and/or other downhole tools.  A standard fracturing operation may involve anywhere from 5 to over 500 sleeves, requiring 1 to more than 100 darts, depending on well length and complexity. These darts must be delivered to the top of the wellhead stack or "wellhead tree" in a timely and safe manner, with accurate tracking to ensure the correct sequence is maintained.  The wellhead tree may contain a series of valves to control pressure from within the wellbore.  Darts can be inserted from the top of the wellhead at atmospheric pressure into an isolated cavity in the top portion of the body of the wellhead after relieving the pressure within the cavity to atmospheric pressure through closing of one or more isolation valve(s).  The dart can be subsequently dropped into the wellbore and exposed to wellbore pressures by reestablishing connection to the wellbore by opening valves in the wellhead.  Conventional methods for dart deployment rely on large physical structures such as stands, stairs, launching barrels, and chutes. These systems require significant on-site setup, staging areas, and personnel presence within the "red zone" (also known as the “hot zone”) areas adjacent to the wellhead—a high-hazard area around the wellhead (typically a 50-100 foot radius) characterized by extreme pressures up to 15,000 psi, volatile chemicals, the potential presence of toxic gases such as hydrogen sulfide (H2S), and heavy machinery.

[0003] Several primary methods are currently employed for dart launching, each with inherent limitations. The most basic approach involves manual delivery, where personnel physically drop darts into the wellhead. This method exposes workers directly to the red zone, posing severe risks, including high-pressure fluid releases that can cause traumatic injuries, chemical burns from substances like hydrochloric acid or exposure to potentially deadly hydrogen sulfide gas (H2S), and mechanical hazards from equipment failures. As a result, this approach would like to be generally avoided due to its significant safety concerns.  A more advanced method utilizes rotating canister systems mounted atop the wellhead. These devices contain multiple chambers (e.g., up to 23 darts or balls) that revolve to align with the wellhead port after each deployment. Key features include hydraulic brakes and conveyor mechanisms to ensure single-item release, compatibility with pressures up to 15,000 psi, and the opportunity for visual monitoring through protective transparent materials like Lexan walls. However, these are not single-dart systems, increasing the risk of sequence errors where an incorrect dart is deployed, which could be catastrophic to a well’s production development. (e.g., premature or failed actuation downhole). Additionally, once depleted, the canister requires manual reloading, necessitating personnel re-entry into the red zone. This interrupts operations, increases downtime, and exposes workers to ongoing safety risks from chemical exposure, pressure hazards, and potential equipment erosion from proppants. Furthermore, these systems are bulky, requiring transportation and setup that add to costs and site footprint.

[0004] Another variant of rotating systems, as described in US9534469B2, employs stacked rotating trays with solid and open portions to support and sequentially drop balls (or darts) via stepper motor-driven rotation. Stationary stops limit object travel, ensuring controlled release as openings align. While compact and potentially jam-free, it shares limitations with canister designs, including the need for manual loading or setup that may involve red zone entry, and potential for misalignment or jams if objects are not precisely sized.  Yet another method employs a pneumatic conveyance system, that transports darts from outside the red zone, along a hose like mechanism, to the top of the frac tree via suction or air pressure. For example, the system described in the publication WO2023197072A1 involves an automated transport tube that minimizes direct handling at the wellhead. While this reduces some red zone exposure by allowing remote loading, it demands substantial setup, including extensive piping and surface area for the conveyance infrastructure. Maintenance is complex due to potential blockages or failures in high-pressure environments, and the system remains susceptible to operational delays from equipment breakdowns.

[0005] Across these conventional systems, common challenges persist, including high equipment rental and transportation costs—often reaching $10,000 or more per day—and substantial setup time that can impede other wellsite activities. They also fail to fully mitigate safety risks: high-pressure hazards (e.g., blowouts or line whips), chemical exposures (e.g., to biocides or NORM-contaminated flowback), mechanical dangers (e.g., crushed-by incidents from heavy manifolds), and fire/explosion potentials from flammable gases remain prevalent in the red zone. Industry statistics underscore these issues, with oil and gas extraction fatality rates four times the national average, and fracking sites reporting over 1,000 incidents annually, many tied to wellhead failures. Case studies, such as the 2020 Wendland 1-H blowout in Texas (resulting in three fatalities from a flash fire in the red zone due to pressure interference) and the 2018 Pryor Trust incident in Oklahoma (five deaths from an uncontrolled gas release), highlight how personnel presence amplifies these dangers. Furthermore, while some systems can operate in inclement weather, they do not fully address human error in dart selection, loading, or maintenance, nor the fatigue from long shifts that impairs judgment in dynamic, high-energy environments.

Safety Considerations in Wellhead Operations

[0006]  To further illustrate the critical need for innovation, wellhead operations during fracking involve continuous high-energy processes that demand rigorous hazard mitigation. Personnel in the red zone face compounded risks: rapid depressurization can cause explosive decompression, inhalation of H2S at concentrations as low as 100 ppm leads to unconsciousness, and proppant abrasion erodes valves, increasing failure likelihood. Additional factors like confined spaces, falls on slippery surfaces, and simultaneous operations across nearby wells exacerbate these threats. Industry shifts toward automation (e.g., remote valves) aim to reduce entry, but manual tasks in prior art persist, necessitating advanced PPE, training, and compliance with regulations like OSHA's hazard recognition standards. Despite these measures, the persistence of red zone requirements in conventional dart delivery underscores a gap in fully remote, personnel-free solutions.

[0007] No known commercial systems in current oil and gas operations utilize the disclosed suspension cable conveyance, UAV-based delivery or crane/boom delivery of darts into the wellbore disclosed below.  Generally known methods of delivery have been previously described and general UAV-related delivery patents focus on packages (e.g., US20160257423A1 for aerial parcel systems or US10457421B2 for drone docking), while known oil/gas UAV applications are limited to inspections, monitoring, or emissions detection. For instance, WO2019229124A1 (deploying micro-drones into wellbores), does not address surface delivery of tools like darts in hazardous environments. Logistics UAVs in other sectors (e.g., medical or e-commerce) exist but lack adaptation to oilfield constraints, such as precise placement amid high winds or rain. This absence of such systems for delivering darts to the wellbore suggests a novel opportunity for the disclosed suspension cable conveyance, UAV-based delivery or crane/boom delivery of darts systems integration in well completions, particularly for eliminating red zone exposure and reducing operational footprints.

[0008] There exists a need for such systems and methods that can deliver darts, balls, or other downhole tools to the wellhead in a safe, efficient, and accurate manner, without requiring any personnel to enter the red zone, thereby mitigating high-pressure, chemical, mechanical, and explosion risks entirely. Such systems should also minimize equipment footprint, reduce operational costs, and ensure precise tool verification and deployment to prevent downhole errors, addressing the limitations of existing manual, canister, and pneumatic approaches.

OBJECTS OF THE INVENTIONS

[0009] Objects of the Inventions include at least the following:

- A primary object of the inventions is to provide enhanced safety.  The disclosed systems and methods eliminate the need for any personnel to enter the red zone during dart delivery, thereby mitigating high-pressure, chemical, mechanical, and explosion risks inherent in conventional methods.

- By overcoming the bulky setup and reloading interruptions of prior art (e.g., canister or pneumatic systems), the disclosed systems and methods reduce equipment footprint, lower operational costs by minimizing rentals and downtime, and ensures accurate tool sequencing through single-item verification.  These systems provide for reliable dart sequencing.  Darts are individually delivered to the funnel drop zone, minimizing the risk of unscheduled dart drops.

- Another object of the inventions is reduced mechanical failure.  The systems’ design ensures dependable performance with minimal risk of mechanical issues.

- Another object of the inventions is to provide flexible deployment options.  The systems key universal components of the dart holder and funnel assemblies are compatible with either a suspension cable system, unmanned aerial delivery and/or crane/boom system.

- The disclosed inventions are rated for deployment under moderate adverse environmental conditions, though operations and weather conditions may dictate use of one system over another (e.g. use of suspension cable system in lieu of UAV in the case of high winds) or a pause in extreme weather to ensure safety and precision; the systems and methods disclosed may be adapted for other downhole tools or industrial applications requiring precise, remote delivery.

- The disclosed systems’ respective abilities to operate under autonomous programming combined with sensor-camera integration represent a novel advancement, enabling reliable performance in oilfield hazards where prior art falls short.

LIST OF FIGURES

The following listed Figures are fully incorporated into this disclosure by reference:

Figure 1- Top, Side and Isometric Views of Exemplar Embodiment of Dart Holder Assembly

Figure 2A and Figure 2B - Component Views of Exemplar Embodiment of Dart Holder Assembly

Figure 3A – Dart and Dart Holder Side-By-Side

Figure 3B – Loading the Dart into the Dart Holder

Figure 3C – Closing and Locking the Release Arm to Hold the Dart in the Dart Holder

Figure 4 -- Flange Mounted Funnel Assembly Attached to Wellhead

Figure 5A -- Dart Holder Assembly is Lowered into Funnel Until Legs Contact Funnel Wall

Figure 5B – Once Command is Given for Release of Dart, Release Arm Opens, Releasing Dart into Funnel

Figure 5C – Released Dart Passes Through Funnel Orifice Into Wellhead

Figure 6 – Side View of Suspension Cable Conveyance System

Figure 7 – UAV with Dart Holder Attached

Figure 8 – Overhead Flight Path of UAV Relative to Wellhead

Figure 9 – Close-up of Camera Attached to UAV

Figure 10 – Flowchart of UAV Conveyance System Operation Steps

Figure 10A – Photo of UAV Preparing to Lower Dart Into Funnel

Figure 10B – Schematic of UAV Over Funnel Flange Connected to Wellhead

Figure 10C – Photo of UAV Lowering Dart Holder Into Funnel

Figure 10D – Photo of UAV Returning After Dart is Released

Figure 11 – General Schematic of Crane-Based Dart Delivery System

SUMMARY AND DESCRIPTION OF THE DISCLOSED INVENTIONS AND EXEMPLARY EMBODIMENTS

[0010] Various illustrative embodiments of the inventions are described below.  For clarity, not all features of the inventions as implemented are described below in the exemplary embodiments.  Such developments of actual embodiments requires numerous specific decisions to achieve the operator’s specific goals, including systems related and business constraints which vary from one application to another.  Furthermore, those of ordinary skill in the art will recognize that such development efforts may be complex and time-consuming,, but would nevertheless be routine for those having the benefit of this disclosure.  The subject matter is described with reference to the referenced figures are illustrative examples of the present disclosure.  Various elements, systems and devices are schematically or illustratively depicted in the figures for purposes of explanation only and so as not to obscure this disclosure with details that are well known to those skilled in the art.  The terms used herein will be understood and interpreted to have a meaning consistent with the understanding of such terms by those skilled in the relevant art.  Unless expressly set forth in the specification in a definitional manner, no special definitions different from the ordinary and customary meanings as understood by those skilled in the art is intended or implied by usage of a term or phrase herein.

The Dart Holder Assembly (a/k/a Dart Launching Tube)

[0011] A key component to the present invention is the dart holder assembly (also referred to as the dart launching tube), which is modular and designed to be employed along with multiple systems and methods to deliver darts to a funnel assembly attached to a wellhead without the need for human intrusion into the red zone.  As shown in FIGS. 1-2B, the dart holder assembly substantially comprises (i) a central cylindrical chamber for holding a dart, (ii) a bottom-loading port for manual loading of the dart, (iii) a release mechanism (e.g. a bottom gate or release valve) that opens remotely to drop the dart into the receiving funnel and (iv) angled alignment “legs” at its base for aligning the assembly over the center of the funnel’s opening discussed below.  The release mechanism located at the bottom of the cylindrical chamber holds the dart in the chamber until a command is provided via radio signal which causes the release motor to actuate and open the release gate or valve (or a magnetic lock or other locking mechanism) allowing the dart to drop through bottom of the cylindrical chamber under the influence of gravity.  An internal camera located as shown in FIGS. 2A-2B to provide visual confirmation of the release of the dart from the chamber into a funnel once the release command is provided.   A dart is loaded into the dart holder chamber through the bottom orifice of the chamber as shown in FIGS. 3A-3C.  The release mechanism arm is opened, a dart is inserted in correct position in the cylindrical chamber with its top end and bottom end aligned with the top end and bottom end of the dart holder chamber.  Once inserted, the release mechanism arm is closed and holds the dart in position until a command is given for the release motor to open the arm (or release a magnetic lock).  As will be appreciated by those skilled in the art, the novel dart holder apparatus disclosed herein may be used in a variety of systems and methods for inserting darts into a wellhead, including those preferred embodiments disclosed herein.

The Funnel Assembly

[0012] Another modular feature of the present invention is the flange mounted funnel assembly which attaches to the wellhead to receive darts and allow their passage into the wellhead as illustrated in FIG. 4 and 5A-5C  The funnel, like the dart holder assembly, is designed to be employed along with multiple dart delivery systems and methods of the present inventions to deliver darts into a wellhead without the need for human intrusion into the red zone.  The flange-mounted funnel (or other receiving device) is welded to a standard wellhead flange and/or bolted to the top of the wellhead, with an interior surface angled to guide the tool into the bore without sticking or inversion.

Systems and Methods for Positioning the Dart Holder Assembly over the Funnell Assembly

- Suspension Cable Conveyance and Dart Delivery System

[0013] The suspension cable conveyance system is designed to remotely deploy darts into a wellhead during fracking or similar operations, while keeping operators outside the red zone.  As illustrated in FIG. 6, the system combines a driven pulley and an idler pulley to provide a circuit for a continuous loop of cable or synthetic fiber to precisely position and control a dart holder assembly (as described above) and additional camera system to visualize deployment of a dart into the funnel assembly (as described above).  Multiple suspension cable conveyance systems, with multiple components, may also be employed for deploying darts to multi-well completion applications.  The suspension cable conveyance system’s dual-pulley design provides both horizontal/vertical conveyance and fine vertical control at the wellhead, enabling precise, repeatable dart launches without manual intervention near pressurized equipment.  In one embodiment a separate module may be used to replace the “legs” on the dart holder with wireless cameras and winch to provide vertical operation above the funnel at the loading areas.  The dart holder’s winch line may be spliced into the suspension cable so as to not inhibit the suspension cables’ ability to run across a guiding pulley or groove.

[0014] As illustrated in FIG. 6, the suspension cable conveyance system is comprised of the following components, in addition to the aforementioned modular dart holder assembly and the funnel assembly.

A.	Suspension Cable Conveyance system

- The driven pulley is mounted at ground level, typically on a trailer, lift, portable platform, or vehicle-mounted hitch assembly.

- It houses a single high-strength line (e.g., steel cable, Kevlar rope, or other rated synthetic line) that extends from the driven pulley to the idler pulley at the top of the wellhead arm mounted on a receiving funnel or other top-side receiver (hereinafter the “suspension cable”).

- The line forms a closed loop that can move bidirectionally—forward (toward the wellhead) or reverse (toward the loading zone).

- The dart holder winch line is attached to the lower portion of this loop and travels with the dart holder as the suspension cable moves.

- The dart holder assembly is fitted with a servo-winch and cameras for suspension cable conveyance.

- Although typical operation ranges are 30–50 ft, the system can convey loads over longer distances.

- The suspension cable can operate at any incline or angle, depending on site layout.

- The system includes integrated limit sensors and tension monitoring to prevent over-travel or over-tensioning.

B.	Dart Holder Cable Conveyance Upfit

- The dart holder assembly may include an upfitting winch (“Upfit”) that serves to permit the primary vertical axis movement and monitoring support, including:

  - A winch system (used for precise vertical movement to position the dart holder assembly into the funnel assembly)

  - A downward-facing camera for visual alignment and monitoring

  - A dart holder assembly (as previously described) for dart deployment

- The Upfit includes optional power and control modules (battery, motor controller, communication receiver).

- The Upfit can accommodate additional attachments or sensors as needed for specific applications (e.g., environmental sensors, pressure gauges, or alignment lasers).

C.	The Upfit Winch System

- The Upfit winch is mounted directly to the dart holder.

- It controls the vertical positioning of the dart holder assembly relative to the wellhead funnel assembly.

- A winch line suspends the dart holder assembly below the suspension cable and enables controlled extension and retraction during the dart release process.

- The winch includes positional feedback and limit switches for automated and repeatable lowering/raising sequences.

D.	Dart Holder Assembly

- The dart holder assembly is as is described above.

E.	Camera and Control System

- A downward-facing camera mounted on the Upfit assembly provides continuous visual feedback to the operator.

- The camera transmits live video to a surface control station for monitoring alignment, lowering, and dart release.

- The control station can be a laptop or industrial PC housed in a climate-controlled trailer or operated remotely via network connection.

- The operator can fully control both winches, monitor line tension, and view camera feeds from this station.

[0015] An exemplary suspension cable conveyance system setup is as follows:

- Site Arrival and Equipment Staging

  - The suspension cable conveyance system trailer or portable platform is positioned in line with the wellhead and outside of the red zone.

  - The receiving funnel assembly (with integrated pulley arm) is mounted securely to the top of the wellhead.

- Pulley Arm Alignment

  - The idler pulley arm is positioned on the far side of the funnel, aligned directly with the intended driven pulley position.

- Driven pulley Mounting

  - The driven pulley is secured using one of the following:

    - Weighted trailer or platform

    - Stakes or ground anchors

    - Vehicle hitch mounting system

  - The mounting must withstand the maximum expected line tension.

- Line Installation

  - The high-strength line is run from the driven pulley through the idler pulley arm, and back to the driven pulley, forming a continuous loop.

  - The dart holder is attached to the lower portion of the line loop.

- Tensioning

  - The line is tensioned using controlled winch actuation or mechanical tensioning devices to achieve proper sag and stability.

- System Power and Control Setup

  - Control electronics, winch controllers, and visual display are powered and connected.

  - Communications with the Upfit camera and the Upfit winch are tested.

  - The system can optionally be connected to a remote-control station for off-site operation.

[0016] An exemplary surface cable conveyance system operation workflow is as follows:

- Dart Preparation

  - The correct dart or ball type is verified and logged.

  - The driven pulley is CNC controlled and oriented as at “start" where the operator can splice the upfit dart holder winch line to the suspension cable.

- Dart Loading

  - A dart is inserted into the launching chamber by the operator (outside of the red zone).

  - The remote gate is closed below the dart to secure the dart in position.

- Suspension Conveyance

  - The operator commands the driven pulley to move the suspension cable forward along loop towards the wellhead funnel assembly.

  - The destination position (above the receiving funnel) is preprogrammed for precise repeatability.

- Alignment Verification

  - Once positioned, the operator visually confirms alignment of the dart holder via the dart holder assembly cameras.

- Dart Holder Assembly Lowering

  - The operator activates the Upfit winch to lower the dart holder assembly.

  - The dart holder assembly descends until its alignment legs contact the upper chamber of the funnel, automatically centering it.

- Dart Release

  - The operator sends a command to open the bottom gate of the dart holder assembly.

  - The dart is released and drops cleanly into the funnel and subsequently into the wellhead.

  - The cameras feed confirms the successful drop.

- Dart Holder Retraction

  - The Upfit winch raises the dart holder assembly back to its travel position above the funnel.

- Return to Start

  - The driven pulley reverses direction, returning the dart holder to the preprogrammed start position near ground level.

  - The system is ready for reloading.

- Repeat

  - The process is repeated as needed for subsequent dart launches throughout the fracking operation.

  - For multi-well completion operations, multi-suspension cable systems may be set up on a trailer handling each individual wellhead.  Alternatively, if all wellheads are in a relatively straight-line configuration, a single suspension cable system may be mounted to direct one or more dart holders to the funnel systems above each wellhead to release the darts in the funnels above each of the wellheads.

[0017] Some optional/supporting features that may be employed with the disclosed suspension cable conveyance system include, without limitation, the following:

- Safety interlocks to prevent movement during loading or release.

- Automated homing and position tracking to ensure repeatable accuracy.

- Integrated sensors (tension, load, or limit sensors) for fault detection.

- Wireless data link between surface control and dart holder for command and telemetry.

- Power redundancy via onboard battery or surface supply.

- Modular attachments for future expansion (e.g., laser alignment, pressure verification, or temperature sensors).

- UAV Dart Delivery System

[0018] The UAV dart delivery alternative system provides a semi-automated aerial delivery system for darts to the wellhead funnel assembly.  As illustrated in FIGS. 7-10D, the system comprises a UAV equipped with a specialized dart holder assembly (a/k/a launching tube) (as described above), and a flange-mounted funnel or other receiving device affixed to the wellhead (as described above). As with the suspension cable system, unlike prior art manual, canister, or pneumatic systems that require personnel in the red zone for loading or maintenance, along with significant equipment prone to difficult setup and breakdowns—this arrangement enables the precise placement of darts directly into the wellhead without any personnel entering the hazardous red zone during deployment, with an operator remotely confirming the dart holder positioning and manually releasing the tool from a safe location outside the red zone and without large equipment.

[0019] In preferred embodiments, the dart holder is configured for single-tool loading and incorporates two remotely actuated mechanisms: a bottom gate for releasing the tool into the funnel or other receiving device and a quick-release coupling for emergency detachment from the UAV.  This single-tool design emphasizes novelty by preventing sequence errors common in multi-chamber prior art systems (e.g., rotating canisters), ensuring mechanical and operational integrity. The dart holder and funnel or other receiving device are adjustable in size and structure to accommodate various darts, balls, or other downhole tooling.  The UAV is rated for payload capacities up to approximately 55 pounds and fitted with a real-time video camera for precise alignment and drop verification. A key novel feature is the pressure sensor on the dart holder, which detects contact with the funnel or other receiving device, forming part of a three-part verification process: initial GPS confirmation of position, sensor detection of physical contact, and final visual confirmation via camera—addressing alignment and error risks not mitigated in prior art conveyance methods. The UAV is designed for operation in adverse conditions and a modular setup allowing for dual UAV systems to ensure redundancy in case of downtime.

[0020] The operational process begins with the verification of the correct tool, which is critical to prevent deployment of an incorrect dart, which could cause downhole errors such as premature actuation or failed isolation, leading to costly interventions. This verification step ties directly to prior art limitations, such as human error in multi-dart canister sequencing, by enforcing single-item checks for enhanced accuracy. The verified tool is loaded into the dart holder, and the UAV autonomously navigates to the wellhead under operator supervision, following a pre-defined flight path marked with red paint or tape to restrict personnel access and enhance safety. Upon arrival, the UAV positions the launching tube over the funnel or other receiving device, with the three-part verification process (GPS position, pressure sensor contact, and camera visual confirmation) ensuring accuracy before release. Novelty is further emphasized in this integrated verification, which provides multi-layered precision unavailable in traditional systems reliant on manual or mechanical alignment. The operator then remotely commands the bottom gate to drop the tool, confirming success via camera. The UAV returns to the loading area for reloading, with the option to immediately release the tube via the quick-release mechanism in emergencies to protect the UAV and maintain operational safety.

- Boom/Crane Robotic and Remote Controlled Dart Delivery System

[0021] In an alternate configuration as exemplarily illustrated in FIG. 11, a dart delivery system can be operated without the suspension cable assembly or UAV, using a boom or crane-based delivery system, which may be controlled manually or remotely by robotics or artificial intelligence.  This configuration provides a means of performing the same launching process while maintaining all functional steps of the dart deployment operation.

[0022] An exemplary boom or crane-based delivery system conveyance system setup is as follows:

- System Description

- The dart holder assembly is attached directly to the hook or lifting arm of a crane or boom system rather than being suspended from pulley system.

- The crane or boom serves as the primary conveyance mechanism, allowing the operator to swing the dart holder assembly between the loading position (near ground level) and the wellhead funnel.

- The system retains all its standard components, including:

  - The dart holder assembly described above, and

  - The funnel assembly described above.

- The lowering and lifting motions are controlled directly through the crane’s hoisting mechanism rather than through Uplift winch.

[0023] The exemplary boom or crane-based delivery system conveyance system operates as follows:

- Operation

- The operator loads the dart into the dart holder at ground level, following the same loading procedure used in the winch-based system.

- The crane operator swings the boom to position the dart holder directly above the receiving funnel mounted on the wellhead.

- The dart holder is lowered by the crane until the angled legs engage and center the dart holder within the funnel.

- Once properly aligned, the operator sends the command the bottom gate release, allowing the dart to drop through the funnel and into the wellhead.

- The crane then raises the launching tube out of the funnel and returns it to the loading position to repeat the process as needed.

[0024] The particular embodiments disclosed above are illustrative only, as the inventions may be modified, amended and practiced in different but equivalent manners apparent to those skilled in the art having the benefit of the teachings in the disclosures herein.  No limitations are intended to the details of construction or design of the elements shown.  It is therefore appreciated that the particular embodiments disclosed above may be altered or modified such that variations of the same are considered within the scope and spirit of the invention.

ABSTRACT

In an illustrative embodiment, the dart delivery system and methods comprise, among other things, a dart holder capable of holding one or more darts, wherein said dart holder is configured to remotely release said darts upon command, a suspension cable system for transporting said dart holder to a wellhead without any human incursion into the redzone area around said wellhead. And a funnel attached to said wellhead configured to receive one or more darts released from said dart holder and to pass said darts into said wellhead.

CLAIMS

What is claimed is:

    - A system for deploying darts into a wellhead in controlled fluid communication with a well for well fracking operations, comprising:

a dart holder capable of holding one or more darts;

wherein said dart holder is configured to remotely release said darts;

a means for transporting said dart holder to a wellhead without any human incursion into the redzone area around said wellhead;

a funnel attached to said wellhead configured to receive one or more darts released from said dart holder and to pass said darts into said wellhead.

    - The system of claim 1 wherein said means for transporting said dart holder comprises a suspension cable conveyance system

    - The system of claim 1 wherein said means for transporting said dart holder comprises an unmanned aerial vehicle (UAV) system.

    - The system of claim 1 wherein said means for transporting said dart holder comprises a boom or crane based system.

    - A method for deploying darts into a wellhead for well fracking operations, comprising:

loading a dart holder with one or more darts;

transporting said dart holder, without any human incursion into the redzone area around said wellhead, through a means and positioning said dart holder over a funnel attached to a wellhead capable of receiving said darts;

remotely releasing said darts from said dart holder into said funnel

    - The method of claim 5 wherein said means for transporting said dart holder comprises a suspension cable conveyance system.

    - The method of claim 5 wherein said means for transporting said dart holder comprises an unmanned aerial vehicle (UAV) system,

    - The method of claim 5 wherein said means for transporting said dart holder comprises a boom or crane system.